# Design Document: RFID Busy State Synchronization

## 1. Objective

To synchronize the bRfidBusy flag with hardware Trigger Press/Release events and UI interactions. This prevents race conditions, duplicate commands, and unintended inventory interruptions caused by trigger bouncing or UI latency.

## 2. Problem Statement

Previously, the bRfidBusy flag was updated only upon receiving INVENTORY_START_EVENT or INVENTORY_STOP_EVENT from the reader. This introduced a latency gap (tens to hundreds of milliseconds) between the user’s action (Trigger Press) and the state update.

Consequences: 1. Race Conditions: Rapid trigger presses (bouncing) could bypass the if(bRfidBusy) check, causing OperationFailureException (Reader already performing). 2. UI Desync: The UI might remain enabled for a split second after starting, allowing double-taps. 3. Interruption: If the busy check logic included a safety stopInventory() call, a trigger bounce could inadvertently stop the inventory immediately after it started.

## 3. Solution: Optimistic State Locking

We implemented an Optimistic Locking strategy where the application assumes the command will succeed and updates the state immediately, reverting only on failure.

### 3.1. Workflow: Trigger Press (Start Inventory)

- Event: STATUS_EVENT_TYPE.HANDHELD_TRIGGER_EVENT (Pressed) received in eventStatusNotify.

- Action: Call performInventory().

- Synchronization (Inside performInventory):

  - Check: If bRfidBusy is true, return immediately (Ignore). Crucial: Do not call stopInventory() here to prevent bounce-induced stops.

  - Lock: Set bRfidBusy = true immediately.

  - Execute: Call reader.Actions.Inventory.perform().

  - Revert: If perform() throws an exception, set bRfidBusy = false and update UI.

- Confirmation:

  - Reader sends INVENTORY_START_EVENT.

  - eventStatusNotify receives event.

  - Set bRfidBusy = true (Idempotent confirmation).

  - Update UI to “Running” state.

### 3.2. Workflow: Trigger Release (Stop Inventory)

- Event: STATUS_EVENT_TYPE.HANDHELD_TRIGGER_EVENT (Released) received in eventStatusNotify.

- Action: Call stopInventory().

- Synchronization:

  - stopInventory() sends the stop command.

  - Note: We do not set bRfidBusy = false yet. The reader is still active until it fully stops.

- Confirmation:

  - Reader sends INVENTORY_STOP_EVENT.

  - eventStatusNotify receives event.

  - Set bRfidBusy = false.

  - Update UI to “Stopped” state (Enable Start buttons).

## 4. Implementation Details

### Modified performInventory Logic

synchronized void performInventory() {
    if (bRfidBusy) {
        return; // Simply ignore, do not stop
    }
    
    // Optimistic Lock
    bRfidBusy = true; 
    
    try {
        reader.Actions.Inventory.perform();
    } catch (Exception e) {
        // Revert on failure
        bRfidBusy = false; 
        if (context != null) context.toggleInventoryButtons(false);
        throw e;
    }
}

### UI Synchronization

The UI state (toggleInventoryButtons) is driven by the asynchronous events to ensure it reflects the actual hardware state, not just the user’s intent.

- INVENTORY_START_EVENT: Disable Start, Enable Stop.

- INVENTORY_STOP_EVENT: Enable Start, Disable Stop.

- Exception in performInventory: Revert to Stopped state immediately.